Evaluation of haptic training: prototype description

Dan Morris, 4/11/2006

This application demonstrates some of the features that we’ve discussed for our “haptic mentoring” experiments, including random trajectory generation and several forms of haptic feedback.  It is not packaged for any experiments yet and does no data logging, but it should start to give us something to discuss in terms of look-and-feel…

There are three sections to this document:

- Interface

- Running a study

- Internal details

- Methods

A note on units:  I’m defining a unit of distance called “chai_units”, which is one unit of distance in our CHAI workspace.  This translates to millimeters in a device-specific way; for a Phantom omni, one chai_unit is 35mm.  We will turn all units into metric units before publication.

For the Omega that was used for the experiment, one chai_unit is 27.27mm.  I confirmed this with a ruler.

Interface

To get started, you should just run the “haptic_training.exe” program in the path where the included dll’s live (i.e. run it from the path you unzip to).  You will see the “ground plane” in red, with a randomly-generated trajectory painted on it.  If you want a new trajectory, you can press “generate trajectory” at any time, as long as the haptic device is currently disabled (the “generate trajectory” function is disabled when the haptic device is enabled).

GL window

You’ll also see three spheres:

- The red sphere represents the position of your haptic device

- The blue sphere represents the current animation position when the trajectory is being animated (I’ll discuss this more later), so it only moves when you press ‘play’.

- The purple sphere is the point on the trajectory that’s closest to your haptic device position (computed using a kd-tree-based nearest-neighbor query).

…we would not necessarily show all these cursors to a test subject.

When you’re ready, press “enable haptics” to turn on the haptic device.  You can move the device around and your nearest position on the curve will be tracked.  You can now turn on one or more of the forces controlled by the checkboxes (make sure to hold on to your device; I’ve tested this with no problems on my omni, but you know how moving a haptic application to a new machine can be a little unpredictable).

Checkboxes

- ‘plane proxy’ just makes the plane a physical object using the standard CHAI proxy.

- ‘plane constraint’ constrains your device to lie on the “ground plane”.  This will be very useful as a test mode; in this mode, we can let the user move along the curve and evaluate how hard they’re pressing in the normal direction (in free-motion mode, without some opposing force, it’s impossible to measure how hard they’re pushing).

- ‘curve constraint’ pulls your device toward the nearest point on the curve.  The best analogy here is that it should feel like the children’s toy in which you slide a bead along a wire.  This may be useful for the same reason as the “plane constraint” mode.

- ‘guidance’ lets the system “pull you along” the trajectory.  If you enable “guidance” and press “play”, you’ll see the blue sphere start moving and you’ll be dragged along with it.  We have not discussed in detail whether there would be a particular time component to our trajectories, or whether the user would move freely along the curve.  Having the trajectory run in a fixed time certainly simplifies the analysis somewhat.

Velocity can vary sinusoidally along the curve, but is currently set to be constant.  Also FYI you can press “play” when haptics are disabled to just see the animation.

- ‘normal force’ enables the rendering of the plane-normal forces that are embedded in the trajectory.  These forces are currently displayed visually as the width of the line.  A thin line is a zero force, a thick line is a positive force.  We have the ability to add negative forces too.  I was limited to a very small range of line widths for my first implementation pass; we can fix that later too.

If you have ‘guidance’ disabled, so there’s no animation, and you enable ‘normal force’ the normal force comes from the “purple point”: the closest point on the curve to your device.  With no other constraints, this is a very difficult mode to use the program in, since you can introduce large force discontinuities.

Sliders

Each of these forces – plane constraint, curve constraint, guidance constraint, and normal forces – is associated with a force constant, which can be controlled from a slider on the right.  Additional sliders control the maximum allowable force (N) and the level of velocity damping (N / (chai_units / s)).

Graphical force indicator

Checking ‘show target force’ or ‘show applied force’ brings up a visual indicator of the current applied force (in green) and/or the current ‘target’ level of force (in blue).  This is intended as a visual training mode, where one might have ‘guidance’ enabled and ‘plane constraint’ enabled, but ‘normal force’ disabled.  Your goal would be to line up the green bar with the blue bar.

The visual indicator travels with the “guide position” so you can focus your attention in one horizontal location.

Running a trajectory

Press ‘play’ to start a trajectory.  If guidance is enabled, a message will appear asking you to move your device to the start position, and you will be gently guided in that direction. This is to avoid initial force transients and to make sure that the user starts a trajectory in the right place.  When you get there, the trajectory will begin.

When the trajectory ends, the guidance force will disappear.  If you press ‘play’ again, the trajectory will play backwards.  This is just a development convenience and we won’t use it in our study.  If you press ‘reset’, the trajectory will start over.

Running a study

Running a study might look like this:

- Have the user fill out a consent form and read the basic instructions

- Start the program

- Walk them through the three training conditions and the test conditions manually; I think this will work better than using the automatic “practice” trials.  Use the following keys to step through the conditions, and use the “play” button to run practice trials:

  - ‘1’: Haptic training

  - ‘2’: Visual training

  - ‘3’: Visual + haptic training

  - ‘0’: Test

- Press ‘start experiment’

- After the trials finish, allow a 3-minute break, then sit the user down and press ‘start experiment’ again

FYI, pressing “start experiment” will automatically do the following:

- Enable haptics, if they’re disabled

- Generate a random sequence of training conditions, based on the contents of the haptic_training.ini file

- Run the user through the relevant “practice” trials if the .ini file indicates that they should be included (I think they should not be)

- Run the trials

Internal Details

Here I’ll describe the algorithms used to generate trajectories and the algorithms used to apply forces to the device.

Trajectory generation

There are three stages to generating a trajectory:

- Generate a curve of x/z positions (y is always 0).  Currently the curve is discretized into 2000 linear segments.

- Generate times for each point on that curve, according to a varying set of (optionally varying) velocities

- Generate forces for each point on that curve

Generating curve positions

- 20 sinusoids are generated, each consisting of randomly-selected DC offsets (in z), amplitudes, frequencies, and phase shifts.

- We step along the x direction, computing the value of each sinusoid and adding them up to get a z value.

- The z values are all then normalized to fit into the desired range (i.e. so they fit on the plane that we show on the screen).

- We now compute the total length of the curve by adding the distances between each pair of successive points

- Then we go through each point to compute its arc-position, i.e. a number from 0.0 to 1.0 that says “how far along the curve is this point?”

All the (x,y,z) points are then stored in a kd-tree for rapid nearest-neighbor queries as we move the haptic probe around later.  The kd-tree is constructed and queried using:

http://www.cs.umd.edu/~mount/ANN/

Generating curve times

In “constant velocity mode” :

- The velocity is constant at 0.6 chai_units / s

- Assign time 0.0 to point 0

- Step along the curve, using the (already computed) arc-distance between each successive pair of points and the constant velocity to assign a time to each point

In “variable velocity mode” (currently disabled) :

- 3 sinusoids are generated, each consisting of randomly-selected DC offsets, amplitudes, frequencies, and phase shifts.  These sinusoids will represent velocity at each point.

- We step along the x direction, computing the value of each sinusoid and adding them up to get a velocity value.

- The velocity values are all then normalized to fit into the desired range.

- We then go through each point and compute the time at which we should reach that point, using the velocity we computed at each point and the distance between each point and its predecessor.

Generating curve forces

- 4 sinusoids are generated, each consisting of randomly-selected DC offsets, amplitudes, frequencies, and phase shifts.  These sinusoids will represent normal force at each point.

- We step along the x direction, computing the value of each sinusoid and adding them up to get a force value.

- The force values are all then normalized to fit into the desired range.

Force computation

The forces corresponding to each ‘check box’ are computed independently, then added up to give a net force.  If the overall force magnitude is greater than the maximum allowable force, it saturates (i.e. it is scaled down to preserve its direction).

Plane proxy

This force is generated from the standard CHAI proxy, using the plane as a mesh.

Plane constraint

A penalty force is generated that tends to move the users haptic probe toward the y=0 plane.  In other words,

fplane constraint = (0,fy,0)

fy = kplane constraint * (0 – device_position.y)

kplane constraint is the value on the GUI’s slider, defined in N / chai_unit .

Normal force

The “current” normal force is looked up in the vector of forces that we computed from random sinusoids, and applied directly to the device.

If we’re playing a trajectory (the usual situation when you’d want to apply the recorded normal force), the “current” normal force comes from the “current” position along the trajectory, regardless of where the user’s probe is.

If the user is moving around but no trajectory is being played, the “current” normal force comes from the closest point on the curve to the user’s probe.  If this is confusing, ignore it; we will almost never use this mode.

Curve constraint

This force constrains the user to move toward the curve.  If dp is the current device position and cp is the closest point on the curve to dp, the force is:

fcurve = kcurve * (cp – dp)

kcurve is defined on the GUI’s slider, and is defined in N/chai_unit .

The closest point on the curve to the current haptic device position is computed as follows:

- Call the user’s probe position P

- All points on the curve (2000 points per curve in this application) are stored in a 3-dimensional kd-tree, along with indexing information that stores the topology of the curve (i.e. they’re not just stored as a point cloud).  Each point is associated with an index that says where it lies on the curve; e.g. the first point is 0 and the last point is 1999.  The notation pi refers to the point with index i.

- At each timestep, we want to find the closest point in the curve to P, so we:

  - Ask the kd-tree for the N points that are closest to P.  I use N=6; higher N cost more time, and lower N can potentially create closest-point errors.  Why not just ask for the ONE closest point to P?  See the figure below for an example.  Here the blue circle represents ‘P’ (the haptic device), and each numbered point is a sample on the curve.  The haptic device should clearly be constrained to the segment between points 1 and 2, but in fact the closest point is point 4.  This is why we have N != 1.

  - For each of the N nearest points pi:

    - Compute the closest point to P on the line segment between pi and pi-1.  In the graphical example below, we would first have retrieved point 4, so we would compute the closet point between P (the blue dot) and the line segments 34 and 45.

    - If either of these line segments store the best closest-point yet, store it.

  - After performing a total of 2N closest-point-on-line-segment computations, we take the best closest-point that we found and call that the current position along the curve.

Guidance force

This force constraints the user to move with the current position in a playing trajectory, i.e. the “guidance ball”.  This force only moves the user in the xz (tangential) plane.  If dp is the current device position and gp is the current guidance position, the force is computed as:

fguidance = kguidance * (gp – dp)

fguidance.y = 0;

kguidance is defined on the GUI’s slider, and is defined in N/chai_unit .

Velocity damping

With all these penalty forces, instability is a real issue in some cases, so we apply a straightforward velocity damping to reduce instability.  Velocity is computed over a 30-millisecond window and a force is applied to oppose the velocity.  If the current velocity is v, the damping force is computed as:

fdamping = -1.0 * kdamping * v

kdamping is defined on the GUI’s slider, and is defined in N/chai_unit .  By default, the magnitude of this force is quite small (imperceptible at “typical” velocities) but has a significant affect on stability.

2.  METHODS

Participants

Twelve right-handed participants, nine male and three female, aged 19 to 21, took part in the present study.  All were undergraduate students.  None had previous experience with haptic devices.  Participants were compensated with a $5 gift certificate, and an additional $10 gift certificate was offered to the three subjects with the highest overall score (across all conditions) as incentive.  Written consent was obtained from all subjects; the consent form was approved by the Stanford University Institutional Review Board.

Apparatus

The haptic device used was a Force Dimension Omega (Force Dimension, Lausanne, Switzerland).  This device was selected because it was able to deliver the sustained forces required for this experiment (up to 8N for up to twenty seconds), which other available haptic devices could not.  Visual information was presented on a 19” LCD monitor placed approximately 2.5’ from the user.  Subjects were able to rest their elbows on a table.

Stimuli

{Dan: Your section on “Internal Details” can be modified and placed here.}

Design

The following 3 training conditions were employed in a blocked design:  haptic display of normal force (H), visual display of normal force (V), and haptic plus visual display of normal force (H+V).  In all three cases, haptic guidance along the trajectory was available so that the participant’s hand was pulled along the trajectory.  The visual display always showed the trajectory as well.  In the H condition, the participant felt the normal force that was perpendicular to the plane where the trajectory lay as s/he moved along the trajectory.  In the V condition, the participant saw a moving display showing the target normal force in blue and his/her applied force in green.  The visual normal-force display was superimposed on the current cursor point along the trajectory so that participant could see both where s/he was on the trajectory as well as the target and applied forces.  The participant’s goal was to adjust the green bar so it was at the same height as the blue bar.  In the H+V condition, both the haptic and visual displays as described above were available to the participant.

The testing condition shows the trajectory visually and applied haptic guidance to move the subject along the trajectory, but provides no information about the correct normal force.  The haptic device is constrained to the xz plane by a spring, which allows the subject to apply the recalled normal force with minimal variation in position.  The subject is instructed to apply the learned normal force pattern.

Procedure

Each participant goes through the following training and testing procedure:

- For each block:

- One of the three training conditions is chosen and a random trajectory is generated.

- The subject is trained on that trajectory using the selected condition.

- The subject is tested on the same trajectory using the testing condition.

- This is repeated twice more per block, for a total of three training trial, interleaved with three testing trials, per block.

The three training conditions are mixed into blocks of twelve trials (containing four of each condition).  Each subject participated in two blocks, with a ten-minute break between blocks, to prevent fatigue.

The device positions and applied normal forces are recorded to disk for later analysis.

Data Analysis

Each testing trial is scored individually for accuracy of force recall.  The input to the scoring mechanism is two force-time curves: the “target” force pattern and the “applied” force pattern.  A simple approach to computing a score might simply subtract the two curves and compute the mean-absolute or RMS difference at each point (or, similarly, compute the area between the two curves).  The following synthetic example illustrates why this is inadequate:

time

force

Here the black curve indicates the “target” force pattern, and the red and green curves indicate two different “applied” patterns.  Clearly the subject applying the pattern of forces shown in red recalled the forces with more accuracy; this subject correctly recalled the amplitude of the two peaks and got the timing of the peaks nearly correct.  The “green subject” did not try to replicate either peak.  But because the peaks of the “red subject” were offset slightly in time, a difference-based metric will not reward his accuracy and will favor the green user here.  To make matters worse, the red subject’s peaks were not shifted by the same amount in time relative to the target peaks, so computing a single time-shift and/or time-scale before differencing (as per [citation]) will still not adequately reward the red subject’s accuracy.

To address this problem, we propose the following scoring scheme based on dynamic programming:

- For each trial, the target and applied force patterns are resampled to a common time base

- The applied force patterns are low-pass filtered by a box filter with a width of 100 milliseconds to remove normal hand tremor.

- Dynamic programming is used to align each sample on the target force pattern to corresponding samples on the applied force pattern.  The input to the dynamic programming algorithm is an error matrix, containing a scalar value that describes how well each point on the target pattern “matches” each point on the applied pattern. If the resampled trajectories are 1000 samples long, this matrix contains 10002 entries.  In other words, each entry in the input matrix answers the question “how bad would it be if these two points were paired with each other?”

For this experiment, each entry in the error matrix is a weighted sum of the RMS difference in forces and the RMS difference in slopes (df/dt values) between the two points being compared.  A penalty value is also specified to the dynamic programming algorithm to penalize time distortions.  Dynamic programming is then used to find an optimal (minimum-cost) pairing between samples on the target and applied curves.  The figure below shows a target force pattern in red, the corresponding applied force pattern in green.  A subset of the pairings computed by DP are indicated in blue.

- The applied curve is warped according to these pairings to lie on the same time base as the target curve.  The figure below shows the same target force pattern in red, but now the warped pattern of applied forces is displayed in black.

- After DP and warping, a score is assigned to each trial as a weighted average of:

  - The DP alignment cost

  - The RMS difference between the two curves after warping

  - The RMS difference between the two curves’ slopes after warping

Weights were adjusted empirically to match visual assessments of recall accuracy without knowledge of the experimental conditions for each of the assessed trials.

- These scores are used to assess the quality of recall; a score of 0 indicates perfect recall.  Larger scores indicate lower recall accuracy.

Results

- Scores are pooled over each training condition, allowing us to compare the recall quality for each training condition (288 recall trials for each condition).  The following figure shows the mean score for each training condition, with standard deviations (+/- 1 SD) in red and 95% confidence intervals in black.

- We see that visual training provides more accurate recall than haptic training, but visuohaptic training provides even more accurate recall than visual training.  A one-way ANOVA confirms a significant difference among groups, p = 3.0789e-006.  T-tests show that haptic training is significantly worse than visual training (p=0.00166) and that visuohaptic training is significant better than visual training (p=0.0112).

We also observe that 9 of 12 subjects had the lowest mean error in the visuohaptic training mode, and only 1 of 12 subjects had the highest mean error in the visuohaptic training mode.  A breakdown of mean scores for each subject is shown here with 95% confidence intervals:

References

Papers indicating the “correct” range of forces:

Choi, S., Walker, L. A., Tan, H. Z., Crittenden, S., & Reifenberger, R. (2005). Force constancy and its role on haptic perception of virtual surfaces. ACM Transactions on Applied Perception, 2(2), 89-105.  (Indicates a “normal” force range of 1-2N).

Chib VS, Patton JL, Lynch KM, Mussa-Ivaldi FA. "The Effect of Stiffness and Curvature on the Haptic Discrimination of Surfaces."  (Indicates an internal force range of 1-2 N (Fig. 3 b and d)).

Haptic learning papers:

# D. Feygin, M. Keehner, and F. Tendick, "Haptic Guidance: Experimental Evaluation of a Haptic Training Method for a Perceptual Motor Skill.'' Proceedings 10th Symposium on Haptic Interfaces for Virtual Environment and Teleoperator Systems, part of IEEE Virtual Reality 2002, Orlando, FL, pp. 40--47, Mar. 2002.

# Todorov, E., P. Shadmehr, and E. Bizzi, Augmented feedback presented in a virtual environment accelerates learning of a difficult motor task. Journal of Motor Behavior, 1997. 29(2): p. 147-158.

# Implementation and Evaluation of a Haptic Playback System,  Robert L. Williams and Mayank Srivastava, Robert Conaster, and John N. Howell, Haptics-e, Vol. 3, No. 3, May 3, 2004.

# Virtual Training for a Manual Assembly Task. R.J. Adams, D. Klowden, B. Hannaford, Haptics-e, Vol. 2, No. 2, October 17, 2001

# Gillespie RB, O’Modhrain M, Tang P, Zaretzky D, Pham C. The Virtual Teacher. Proceedings of the 1998 ASME International Mechanical Engineering Congress and Exposition, Nov 15-20 1998. 1998. Anaheim, CA, USA: ASME, Fairfield, NJ, USA.

# Patton, J.L. and F.A. Mussa-Ivaldi, Robot-Assisted Adaptive Training: Custom Force Fields for Teaching Movement Patterns.  IEEE TRANSACTIONS ON BIOMEDICAL ENGINEERING, VOL. 51, NO. 4, APRIL 2004

# A haptic rendering technique based on hybrid surface representation.  L Kim (KIST), G Sukhatme, M Desbrun (USC).  IEEE CG and applications, March 2004.  Special issue on haptics.

# Teaching to Write Japanese Characters Using a Haptic Interface.  J Solis, C Avizzano, M Bergamasco.  Haptics 2002.

# Toward Machine Mediated Training of Motor Skills: Skill Transfer from Human to Human via Virtual Environment.  Y Yokokohji, R Hollis, T Kanade.  Kyoto U / CMU.  5th IEEE International Workshop on Robot and Human Communication, November, 1996

# Haptic Teaching Using Opposite Force Presentation.  Satoshi Saga, Naoki Kawakami, Susumu Tachi.  University of Tokyo, Japan.  Poster at WorldHaptics 2005.

# Motor Skill Training Assistance Using Haptic Attributes G Srimathveeravalli, K Thenkurussi.  VRLab, SUNY Buffalo.  WorldHaptics 2005.

Papers discussing curve matching:

Performance Evaluation Metrics and Statistics for Positional Tracker Evaluation, C Needham and Roger Boyle, Leeds (UK), ICVS 2003 (Computer Vision Systems Third International Conference)

Continuous Dynamic Time Warping for translation-invariant curve alignment with applications to signature verification.  Mario Munich, Pietro Perona, CalTech, ICCV ’99.

Partial Shape Matching for CBIR of Spine X-ray Images, S Antani, X Xu, L Long, G Thomas, NLM and BYU, SPIE Electronic Imaging, 2004.

Euripides G. M. Petrakis, Aristeidis Diplaros, Evangelos E. Milios: Matching and Retrieval of Distorted and Occluded Shapes Using Dynamic Programming. IEEE Trans. Pattern Anal. Mach. Intell. 24(11): 1501-1516 (2002).

Anecdotal Observations

- Our strategies in all cases seem to involve discretizing our memory of the force trajectory into “high” and “low” regions, so we’re pretty good at getting the shape of the peaks and valleys.  I’m not very good at getting the slopes right in transitions from low to high; it’s very hard to even perceive the subtle changes in force that represent slope.

- There is an interesting confound when you first look at the screen with no instructions; you really want to believe that the on-screen trajectory (which has nothing to do with force) is what you’re feeling.  Dissociating the force components is an interesting problem that may have some interesting results unto itself.

- Our original “visual” condition involved pressing against the plane to make the two bars match up.  Effectively, this was not only visuohaptic, it was actively visuohaptic, and the active nature of the task may have had additional benefit.  Exploring active vs. passive haptic learning may be interesting unto itself.

- It’s actually interesting how we’ve all been using Phantoms for years and years and never noticed how low the force limits are.  4.0N – which is not very much at all, and even feels like not very much at all –pretty much makes the Phantom give up when sustained for five seconds.  It says something interesting about the way we use haptics that we are used to feeling much higher forces than this, but almost never sustain them for even five seconds at a time.

- It is difficult at times to dissociate the tangential from normal forces that I’m feeling during haptic training, particularly for the z (in and out of the monitor) direction.  My strategy is something like “hold the Phantom in place and feel what force I get”, and when the ball is moving in the plane, I still get a lot of force.  I wonder how the results would differ if we had some trajectories with no force in the z direction.  This may also be a function of where the Phantom is relative to my body, which raises the question of how we tend to define primary axes of movement.