Subject Instructions for Haptic Mentoring Experiment

Dan Morris, 2/6/06

Bold text contains instructors for the experimenter; all other text should be read aloud.

- Have the subject fill out the consent form

- Have the subject fill out the front of the questionnaire

- Give the subject their Jamba Juice card (tell them it’s a $5 card)

- If you need to log into the machine, your login/password are tchang and changt

Make sure the Omega is connected by running “Haptic Desk”.  If it says device ID 0, type Omega, etc., you’re good to go.  Otherwise power down the Omega and power it back up; this is your first solution for all problems related to the Omega.  The blue light on the Omega that’s closest to the user should be on, the other light on the Omega should be off, and the Omega should be sitting in its calibration position.  If the blue light is ever flashing, it means the device needs calibration, meaning it should be returned to its home position with the wrist fitting into the little hole in the base.  Remember that you generally should hold on to the device; if you ever want to put it down, you should press the black button to turn off the motors.  The light closest to the button tells you whether the motors are on.

Sit down at the experiment station, where you (the experimenter) can reach the device for demonstration.  Have the subject sit next to you, where he can see the screen and the device.  Start haptic_training.exe .

Today we’re going to do an experiment to evaluate the use of haptics to teach specific motor skills.  I’m going to give you a demonstration and explain the procedure before we get started; let me know if you have any questions.

If you look at the screen, you’ll see a two-dimensional pattern that looks like a wavy line in space.  When we begin the experiment, the haptic device will gently pull your hand along this line.  Your hand will move parallel to the table, like this.  [Press ‘0’ to put the program in ‘test practice’ mode, and press ‘play’ to execute the trajectory, then press the black button on the Omega to turn its motors on].

You will be required to push the device down toward the table as you move along the line.  The amount of force with which you’re asked to push down varies as you move along the line.  Your job is to learn those correct forces.

You’ll be learning the correct series of forces in one of three different ways for each movement pattern that you see.  Regardless of which of the three ways you learn the forces for a given movement pattern, every single time you’re presented with the correct forces, you’ll be immediately asked to demonstrate the forces that you learned.  I’m going to show you how that works in just a minute; first I’m going to show you the three ways you’ll learn the correct forces.

One way that you’ll learn the correct forces is through force feedback.  [Press ‘reset’, then press ‘1’ to put the program in ‘force training practice’ mode].  In this training paradigm, you’ll be pulled along the line and the haptic device will push your hand away from the table.  The amount of force you need to apply to keep the device from moving up or down at all is the correct force.  [Press ‘play’ to execute the trajectory].  As the device is being pulled along the line, I’m trying to keep the device from moving up or down and I’m trying to remember how hard the device is pushing my hand.  The screen will always say “force practice” to tell me that I’m in this mode.  [point out the label area].

Another way that you’ll learn the correct forces is through visual feedback.  [Press ‘reset’, then press ‘2’ to put the program in ‘visual practice’ mode].  In this training paradigm, you’ll be pulled along the line, and you just need to watch the blue bar to remember how much force to apply at each point.  [Press ‘play’ to execute the trajectory].  The higher the bar is, the harder you need to push at that point.  The screen will always say “visual practice” to tell me that I’m in this mode.  [point out the label area].

The third way that you’ll learn the correct forces is through a combination of visual and force feedback.  [Hold on to the device, press ‘reset’, then press ‘3’ to put the program in ‘visual/haptic practice’ mode].  In this training paradigm, you’ll be pulled along the line, and you’ll see the same graphic representation that you saw in the visual practice mode.  [Press ‘play’ to execute the trajectory].  The difference now is that you’ll also see a representation of how hard you’re pushing; that’s the green bar.  When you’re pushing with the correct force, the green and blue bars will line up.  The screen will always say “visual/force practice” to tell me that I’m in this mode.  [point out the label area].

Regardless of which of the three ways you learn the forces for a given movement pattern, every single time you’re presented with the correct forces, you’ll be immediately asked to recall the forces that you learned.  We’ll call this “test mode”, and the window will always say “test mode” to remind you that you’re supposed to be demonstrating the forces that you learned.  This is what “test” mode will look like.   [Hold on to the device, press ‘reset, then press ‘0’ to put the program in ‘test’ mode].  [Press ‘play’ to execute the trajectory].  My job right now is to press down against the device, toward the table, according to the forces that I learned.  It’s also useful to note that the device can only push with a certain amount of force, so if I try to push too hard, this little light will go on [demonstrate and point] to tell me that the device is pushing as hard as it can.

For each pattern of movement – that is, each wavy line you see on the screen – you’ll be presented with the correct forces four times, and after each time you’ll be put into ‘test’ mode to demonstrate the forces you learned.  After this sequence, you’ll move onto a new pattern of movement and a new learning paradigm.  In about fifteen minutes, it will tell you that the experiment is finished, at which point we’ll take a short break, then we’ll do another fifteen minutes after that.

Do you have any questions?

[answer their questions if they have any]

As an extra incentive, the three people who do the best job recalling the correct forces will each get an additional $10 in Jamba Juice cards.

If they have no questions, sit the subject down at the station and move the device to a place where he can reach it with his dominant hand.  Press ‘start experiment’ to start the experiment.  The experiment should run for fifteen minutes, then stop.  At that point, press ‘start experiment’ again.

If the subject needs a break before the allotted time is up, have them finish the pass (and ideally the whole trial) that they’re working on, and when it next says “move your device to the start position”, just press the black button to disable the motors and put the device down; it will happily wait forever if you don’t move the device to the start position.  Breaks should be discouraged if possible, as it disrupts the flow of a block of trials.

After the experiment:

- Have the subject fill out the back of the questionnaire

- All the data files from this subject will be in the haptic_training directory, which will be linked from the desktop.  They will be called training_experiment*.*.  Create a new directory named as this subject’s first name, and move all the data files into that directory.  If you don’t do this, it’s not a big deal; the program gives unique filenames to new data files so it won’t over-write old data.  But moving each subject to his or her own directory now will help keep the main directory clean.